|  | POLICY & PROCEDURE PP-009 - Excavation and Underground Services Procedure |
| --- | --- |

| Document | Excavation and Underground Services Procedure | Document No. | PP-009 |
| --- | --- | --- | --- |
| Company | Austen Landscaping & Earthmoving | Revision | 1.0 |
| Prepared By | John Austen | Date | 15/05/2026 |
| Project | WR1773121 MILC Millchester Substation Civil Works | Status | Approved for use / Uncontrolled when printed |

PURPOSE

To control excavation hazards, excavation collapse risk and underground service strike risks.

SCOPE

Applies to excavation, trenching, potholing, backfill, service locating, conduit installation and works near underground services.

RESPONSIBILITIES

| Role | Responsibility |
| --- | --- |
| Management | Ensure suitable SWMS, permits and resources are available. |
| Supervisors | Confirm DBYD/service information, permits, locating, potholing and controls before excavation. |
| Workers | Follow excavation controls, report unidentified services and stop work if conditions change. |

PROCEDURE / REQUIREMENTS

Review DBYD / Before You Dig and site service drawings before excavation.

Mark and positively identify services before mechanical excavation.

Use potholing or vacuum excavation where services may be present.

Do not mechanically excavate within restricted distances unless approved controls are in place.

Excavations must be benched, battered, shored or engineered where required.

Spoil must be placed clear of excavation edges.

Barricades and exclusion zones must be installed where required.

Stop work immediately if unidentified services, marker tape, conduits or unexpected ground conditions are found.

RECORDS / FORMS

MLCH-02 Working Near Underground Services

MLCH-05 Excavations in a Live Substation

MLCH-06 Vacuum Excavation

Permits / service plans

REVIEW

This document shall be reviewed annually, following an incident, when work activities change, when legislation or client requirements change, or when the document is identified as no longer effective.

THINK SAFE - WORK SAFE - GO HOME SAFE

Austen Landscaping & Earthmoving | ABN 30 643 517 350 | Phone: 0407 763 435 | Email: kathieshome@westnet.com.au | Uncontrolled when printed