This SWMS must be reviewed with all workers before work commences. Work must stop immediately if controls cannot be maintained or conditions change.

| Activity / Task | Moving Powered Plant at the Workplace | Location | MILC - Millchester Substation / Project Specific |
| --- | --- | --- | --- |
| Scope | Applies to all Austen workers operating, directing, spotting, or working near powered mobile plant. | SWMS No. | MLCH-04 |
| Training & Experience | Competent and experienced workers, licensed / verified operators and site inducted personnel. | Prepared By | John Austen |
| Site Risk Assessment By | Site Supervisor: John Austen | Creation / Review Date | 15/05/2026 |
| Revision | A | Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |

PURPOSE

This SWMS sets out the hazards, risks and controls for moving powered plant and working around powered mobile plant. It is designed for civil, earthworks, utility and substation work environments, including areas where workers, vehicles, services and plant may interact.

CRITICAL RISKS AND MANDATORY CONTROLS

| Critical Risk | Possible Outcome | Mandatory Controls |
| --- | --- | --- |
| Plant and pedestrian interaction | Crush injury, struck-by injury or fatality | Establish exclusion zones; separate people and plant; use spotters where visibility is restricted; maintain positive communication. |
| Reversing or blind spots | Worker or vehicle collision | Use reversing alarms, flashing beacons, mirrors/cameras and spotters. Do not enter operating zones without operator approval. |
| Plant rollover or instability | Serious injury, plant damage or fatality | Use plant suitable for the task and ground conditions; operate within manufacturer limits; maintain safe speeds; use seatbelts where fitted. |
| Faulty plant or equipment | Mechanical failure, hydraulic injection, fire or injury | Complete daily pre-starts; report defects immediately; remove unsafe plant from service until repaired. |
| Hydraulic hose failure | High-pressure fluid injection injury | Never use hands to check hydraulic leaks; cease work and notify supervisor if leaks are suspected. |
| Refuelling | Fire, explosion, spills or skin/eye contact | Refuel in ventilated area by competent person; turn plant off; allow hot surfaces to cool; have extinguisher and spill kit available. |
| Oil or fuel spills | Environmental harm and slip hazard | Maintain suitable spill kit onsite; contain spills immediately; dispose of contaminated waste correctly. |
| Changing site conditions | Unidentified hazards or unsafe work | Stop work, reassess and update controls before recommencing. Review SWMS when conditions change. |

PPE REQUIREMENTS

| PPE Item | Requirement |
| --- | --- |
| Medium impact eye protection | Required as applicable to task and site conditions. |
| Safety helmet | Required onsite and around plant movement areas. |
| Safety footwear | Required at all times onsite. |
| Hi-vis long sleeve cotton clothing | Required for visibility around mobile plant. |
| Task specific gloves | Durable nitrile or rigger gloves as required. |
| Hearing protection | Required where plant noise exposure is present. |
| UV protection and 30+ sunscreen | Required for outdoor works. |
| Respiratory protection | As required by dust, fumes or site risk assessment. |

SEQUENCE OF WORK AND CONTROL MEASURES

| Sequence of Work | Hazards / Risk | Initial Risk | Work Methods & Control Measures | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning, preparation and verification of competency | Legislation breach; untrained operators; unsuitable plant | Medium | ▪ All workers must hold current construction induction and complete site induction. ▪ Operators must be competent, licensed or verified for the plant being used. ▪ Supervisor to confirm relevant qualifications, VOCs, plant registration and site requirements. ▪ Plant must be suitable for the task, ground conditions and operating environment. ▪ Manufacturer operating manuals, load charts and data plates must be available and readable. ▪ Relevant SWMS must be reviewed where plant interfaces with excavation, underground services, traffic or live electrical assets. | Low | Supervisor |
| Pre-start, consultation and communication | Poor communication; unclear work method; changed site conditions | Medium | ▪ Attend Austen daily pre-start meeting before commencing work. ▪ Discuss plant movements, exclusion zones, travel paths, reversing areas, blind spots and emergency arrangements. ▪ Agree communication methods between operators, spotters and ground workers. ▪ Identify additional hazards and implement controls before work starts. ▪ Workers must report hazards and stop work if unsafe conditions arise. | Low | Supervisor / All Workers |
| Plant inspection and defect management | Faulty plant; mechanical failure; unsafe operation | High | ▪ Plant must be inspected on arrival to site and inspection recorded. ▪ Operators must complete daily pre-start inspection before use each shift. ▪ Visual inspection to be completed if operators change during the shift. ▪ Check lights, beacons, reversing alarms, horn, mirrors/cameras, seatbelt, hydraulics, tyres/tracks and fire extinguisher. ▪ Faults, malfunctions or damage must be reported immediately. ▪ Unsafe plant must be removed from service until repaired or replaced. | Low | Supervisor / Operator |
| Establish work area and exclusion zones | Unauthorised access; workers entering operating zone; public exposure | High | ▪ Erect barricades, cones, signage and exclusion zones where required. ▪ Separate workers, pedestrians, public and vehicles from operating plant. ▪ Workers must not enter plant operating zones until positive contact is made with the operator and permission is given. ▪ Monitor the work area for unauthorised persons. ▪ External parties entering controlled areas must understand the work process and site rules. | Low | Supervisor / All Workers |
| Operating powered mobile plant | Struck-by injury; crush injury; rollover; collision | High | ▪ Powered mobile plant must only be operated by competent and authorised personnel. ▪ Seatbelts must be worn where fitted. ▪ Maintain safe speed for task, ground conditions and work area. ▪ Use flashing beacons, reversing alarms and communication devices. ▪ Personnel working near excavators must remain outside the swing radius and exclusion zone unless authorised. ▪ Where working with an excavator, maintain a minimum distance of 1.5 times the radius of the excavator boom swing unless controlled by spotter and positive communication. ▪ Never allow persons to stand under loads, near crush points or within blind spots. | Low | Operator / All Workers |
| Use of spotters | Restricted visibility; reversing; plant near services or structures | High | ▪ Use a spotter where visibility is restricted, when reversing, or where plant operates near workers, services, structures or exclusion zones. ▪ Spotter must remain in clear view of operator and maintain positive communication. ▪ If visual or radio communication is lost, plant movement must stop immediately. ▪ Spotters must not place themselves in the line of fire or between plant and fixed objects. ▪ Hand signals or communication method must be agreed before work starts. | Low | Spotter / Operator |
| Hydraulic hoses and mechanical components | High-pressure injection injury; pinch points; fluid leaks | High | ▪ Do not use hands to move or check hydraulic hoses while under pressure. ▪ If a high-pressure hydraulic leak is suspected, cease work and notify supervisor. ▪ Use a tool, timber or other safe method to avoid contact with pressurised hoses. ▪ Keep body parts clear of pinch, crush and articulation points. ▪ Defective hydraulic hoses or components must be tagged out and repaired before use. | Low | Operator |
| Refuelling mobile plant | Fire; explosion; skin/eye contact; spill to ground | High | ▪ Refuelling must be completed by a competent person in a well ventilated area. ▪ Plant must be turned off before refuelling. ▪ Do not refuel near ignition sources or hot surfaces until surfaces have cooled. ▪ Fire extinguisher must be available. ▪ Suitable spill kit must be nearby. ▪ Wear PPE as required by SDS, including gloves, safety glasses and durable clothing. ▪ No refuelling within 30 metres of a watercourse unless approved controls are in place. | Low | Operator |
| Oil, fuel or fluid spill response | Environmental harm; slip hazard; contamination | Medium | ▪ Maintain a suitable spill kit onsite for the size of plant and type of fluids used. ▪ Contain spills immediately if safe to do so. ▪ Prevent spills entering drains, waterways or soil. ▪ Dispose of contaminated waste in accordance with site environmental requirements. ▪ Report environmental incidents immediately to supervisor. | Low | Supervisor / Operator |
| Task observation and monitoring | Unidentified hazards; unsafe work; work outside SWMS scope | Medium | ▪ Supervisor to visually check plant is being operated safely and in accordance with manufacturer requirements. ▪ Supervisor to monitor seatbelt use, exclusion zones, communication and spotter controls. ▪ All workers must monitor for additional hazards. ▪ If a new hazard is identified, stop work, make safe and notify supervisor. ▪ Review and update SWMS or controls if work conditions change. ▪ Do not recommence work until hazards are controlled. | Low | Supervisor / All Workers |
| Completion and clean up | Poor housekeeping; unsecured plant; unauthorised use | Medium | ▪ Park plant in a safe designated area with attachments lowered and secured. ▪ Switch off plant, isolate keys and secure where required. ▪ Clean rubbish, materials and trip hazards from work area. ▪ Secure loose materials against wind or weather. ▪ Return permits, inspections and pre-start records where required. ▪ Maintain barricades and signage until area is safe for access. | Low | Supervisor / All Workers |

ENVIRONMENTAL AND SITE CONTROLS

▪ Spill kits must be available where plant, fuel, oil or chemicals are present.

▪ Dust suppression to be used where plant movements generate dust.

▪ Erosion and sediment controls to be maintained where required.

▪ Waste must be placed in correct bins or approved disposal areas.

▪ Plant must avoid unnecessary damage to vegetation, drains, services and site controls.

▪ Weed hygiene controls to be maintained where required by the site or client.

PROJECT INFORMATION AND REQUIREMENTS

| Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |
| --- | --- |
| Workers | 2+ or as required by task |
| Competency | General Safety Induction Card, site induction, task instruction, operator competency / VOC and relevant licences where required |
| Training | Austen Site Specific Induction, toolbox talks, daily pre-starts and Energy Queensland / site inductions as required |
| Supervision & Monitoring | John Austen / project supervisor to monitor work, inspect work areas and review controls |
| Plant & Equipment | Excavator, bobcat, truck, crane, roller, grader, compactor and other project specific mobile plant |
| Tools and Equipment | Radios, barricades, cones, signage, spill kits, fire extinguisher, hand tools and plant pre-start checklist |
| Chemicals | Diesel, petrol, oil, grease, hydraulic fluid or other substances associated with plant use |
| PPE | Refer to PPE section and site requirements |
| SWMS Review | Reviewed by John Austen during works and updated when conditions change |
| Legislation & Codes of Practice | WHS Act 2011; WHS Regulation 2011; Managing Risks of Plant in the Workplace 2021; Managing Noise and Preventing Hearing Loss at Work 2021; How to Manage WHS Risks 2021; WHS Consultation, Cooperation and Coordination 2021; Electrical Safety Act 2002; Electrical Safety Regulation 2013; Environmental Protection Act 1994; Biosecurity Act 2014 |

EMERGENCY AND INCIDENT RESPONSE

▪ Stop work immediately.

▪ Make the area safe if it is safe to do so.

▪ Notify John Austen / Site Supervisor.

▪ Provide first aid or contact emergency services if required.

▪ Isolate unsafe plant or equipment and preserve the incident scene where required.

▪ Report all incidents, hazards, injuries, near misses, property damage and environmental incidents immediately.

▪ Do not restart work until the hazard is controlled and the SWMS has been reviewed if required.

WORKER SIGN ON

We, the undersigned, confirm this Safe Work Method Statement has been explained, its contents are clearly understood and we will comply with the stated requirements. We also confirm that we have the required qualifications to undertake these activities.

| Name | Signature | Date |
| --- | --- | --- |

SUPERVISOR DECLARATION

I confirm I have trained all above workers in this SWMS and I hereby confirm that the documented precautions, controls and work methods will be complied with.

| Name | Position | Date | Signature |
| --- | --- | --- | --- |
| John Austen | Site Supervisor |  |  |

THINK SAFE - WORK SAFE - GO HOME SAFE