|  | SAFE WORK METHOD STATEMENT (SWMS) MLCH-07 - TRAFFIC MANAGEMENT / WORKING NEAR ROADS |
| --- | --- |

This SWMS must be reviewed with all workers before work commences. Work must stop immediately if controls cannot be maintained or conditions change.

| Activity / Task | Traffic Management / Working Near Roads and Vehicle Access Areas | Location | MILC - Millchester Substation / Project Specific |
| --- | --- | --- | --- |
| Scope | Applies to Austen workers managing or working near traffic, site access, delivery vehicles, public roads, internal access roads and plant / pedestrian interfaces. | SWMS No. | MLCH-07 |
| Training & Experience | Competent, trained and site inducted workers. Accredited traffic controllers / traffic management implementers required where specified by the TMP or road authority. | Prepared By | John Austen |
| Site Risk Assessment By | Site Supervisor: John Austen | Creation / Review Date | 15/05/2026 |
| Revision | A | Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |

PURPOSE

This SWMS sets out the hazards, risks and controls for traffic management, site access, vehicle movements, delivery activities and work near roads or internal access routes. It is designed for civil, earthworks, utility and substation work environments where workers, mobile plant, public vehicles, delivery vehicles and pedestrians may interact.

SCOPE

Work near public roads, access roads, road reserves, driveways and site entry points.

Establishment, maintenance and removal of cones, signs, barricades and traffic controls.

Managing site access, delivery vehicles, reversing vehicles and plant movement.

Working near mobile plant, pedestrians, visitors, subcontractors and public interface areas.

Traffic control plan / traffic management plan implementation where required.

Emergency access, parking, loading, unloading, clean up and task completion.

CRITICAL RISKS AND MANDATORY CONTROLS

| Critical Risk | Possible Outcome | Mandatory Controls |
| --- | --- | --- |
| Vehicle or plant striking worker | Serious injury or fatality | Separate people from vehicles and plant; use exclusion zones, spotters, positive communication and hi-vis PPE. |
| Public traffic interface | Vehicle collision, public injury or property damage | Traffic management plan where required; signage, cones, barriers, speed control and authorised traffic controllers. |
| Reversing vehicles | Crush injury or collision | Use spotters where visibility is restricted; reversing alarms; agreed communication; stop if visual contact is lost. |
| Uncontrolled site access | Unauthorised entry, vehicle conflict or public exposure | Control entry points, sign-in requirements, barricades, signage and designated routes. |
| Poor visibility / lighting | Collision, struck-by injury or pedestrian impact | Use hi-vis clothing, beacons, lighting, reflective signage and cease or modify work if visibility is unsafe. |
| Delivery and unloading interface | Crush injury, dropped load or traffic obstruction | Plan delivery route, establish exclusion zone, use spotter and keep workers clear of loading / unloading zone. |
| Working near live services or substations | Electrocution, arc flash or service damage | Review MLCH-02, MLCH-03 and MLCH-05 where plant or vehicles operate near live assets or underground services. |
| Emergency access blocked | Delayed emergency response | Maintain clear emergency access, gates, roads and assembly routes at all times. |
| Road user confusion | Traffic incident or near miss | Install signs and controls in correct sequence; remove or cover redundant signs; inspect controls regularly. |
| Changed site conditions | Uncontrolled hazards or unsafe traffic flow | Stop work, reassess, update controls and communicate changes before recommencing. |

HAZARD IDENTIFICATION

| Hazard Group | Hazards Identified |
| --- | --- |
| Traffic / Vehicles | Moving vehicles, reversing vehicles, delivery trucks, public traffic and uncontrolled access. |
| Mobile Plant | Excavators, bobcats, trucks, rollers and plant moving through access routes. |
| People Interface | Workers, visitors, subcontractors, pedestrians, public and plant operators working in close proximity. |
| Gravity | Slips, trips, falls, uneven ground, falling into open excavations or drains. |
| Mechanical / Kinetic | Crush points, struck-by hazards, moving loads, loose materials and vehicle impact. |
| Environmental | Dust, mud, poor road surface, wet conditions, glare, lighting and weather. |
| Electrical | Traffic or plant operating near live electrical assets, substations, overhead lines or underground services. |
| Communication | Poor signals, unclear directions, radio failure or language / instruction misunderstanding. |
| Emergency | Blocked access, delayed evacuation, obstructed assembly point or restricted emergency vehicle access. |

HIGH RISK WORK INTERFACE

Traffic management and work near roads may interface with high risk construction work, including powered mobile plant, excavation, work near electrical installations/services, live substations, public roads, loading/unloading and work adjacent to underground services. Relevant SWMS must also be reviewed where these risks apply.

PPE REQUIREMENTS

| PPE Item | Requirement |
| --- | --- |
| Hi-vis long sleeve clothing | Required at all times when working near traffic, plant or site access routes. |
| Safety helmet | Required onsite and when working near plant, overhead hazards or loading areas. |
| Safety footwear | Required at all times onsite. |
| Medium impact eye protection | Required as applicable to dust, plant movement, tools or site conditions. |
| Task specific gloves | Required when handling signs, cones, barricades, chains, gates or materials. |
| Hearing protection | Required where plant, vehicles or equipment create noise exposure. |
| Respiratory protection | Required where dust, fumes or exhaust exposure cannot be adequately controlled. |
| UV protection and 30+ sunscreen | Required for outdoor works. |
| Wet weather / night controls | Reflective clothing, lighting or additional PPE where required by site risk assessment. |

RISK RATING GUIDE

| Risk Rating | Required Action |
| --- | --- |
| Extreme | Do not proceed. Alternative method required. |
| High | Senior management / supervisor approval required before work proceeds. |
| Medium | Management responsibility must be specified and controls monitored. |
| Low | Work may proceed with normal supervision and routine controls. |
| Stop Work | All workers have authority to stop work if controls cannot be maintained. |

SEQUENCE OF WORK AND CONTROL MEASURES

| Sequence of Work | Hazards / Risk | Initial Risk | Work Methods & Control Measures | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning, preparation and competency | Untrained workers; incorrect traffic controls; no TMP | High | • All workers must hold current construction induction and complete site induction. • Determine whether a Traffic Management Plan (TMP), Traffic Guidance Scheme (TGS), permit or road authority approval is required. • Accredited traffic controllers / traffic management implementers must be used where required. • Review work area, access points, road speed, traffic volume, pedestrian movements and plant interaction. • Review MLCH-04 where mobile plant is involved and MLCH-02 / MLCH-05 where underground services or excavation is involved. | Low | Supervisor |
| Pre-start and consultation | Poor communication; unclear roles; changed conditions | Medium | • Attend Austen daily pre-start meeting before commencing work. • Discuss traffic routes, exclusion zones, delivery movements, reversing areas, public interface, emergency access and stop-work triggers. • Confirm who is supervising, spotting, controlling traffic and communicating with operators. • Identify additional hazards and implement controls before work starts. • Stop work if the traffic arrangement changes or controls cannot be maintained. | Low | Supervisor / All Workers |
| Site inspection and traffic assessment | Unexpected vehicle movement; poor access; unsafe road conditions | High | • Inspect access roads, gates, turning areas, parking areas and loading zones. • Identify blind spots, tight corners, slopes, soft ground, overhead hazards and pedestrian paths. • Consider weather, dust, glare, lighting, road condition and visibility. • Confirm emergency vehicles can access the site. • Modify work method if access or road conditions are unsafe. | Low | Supervisor |
| Establish traffic controls | Public exposure; vehicle collision; road user confusion | High | • Install cones, signage, barricades, delineation and speed controls in accordance with approved TMP / TGS where required. • Install controls before workers enter traffic exposed areas where practicable. • Use authorised traffic controllers where required by TMP, road authority or site conditions. • Ensure signs are visible, clean, stable and appropriate for the work. • Remove or cover redundant signs to avoid confusion. | Low | Supervisor / Traffic Controller |
| Site access and entry control | Unauthorised entry; uncontrolled vehicles; public interface | High | • Maintain designated entry and exit points. • Visitors and delivery drivers must report to site supervisor / sign in as required. • Separate site traffic from public traffic where practicable. • Use gates, barriers, signage or spotters to control access. • Keep access roads clear of parked vehicles, materials and obstructions. | Low | Supervisor / All Workers |
| Working near moving vehicles | Struck-by injury; crush injury; vehicle collision | High | • Workers must remain clear of moving vehicles unless controls are in place. • Wear hi-vis clothing and remain visible to drivers and operators. • Never stand behind reversing vehicles or between vehicles and fixed objects. • Maintain exclusion zones and pedestrian separation. • Do not cross traffic paths unless it is safe and communication is confirmed. | Low | All Workers |
| Reversing vehicles and plant | Blind spots; crush injury; collision | High | • Avoid reversing where practical by planning one-way routes and turning areas. • Use spotters where visibility is restricted or where people, services, structures or plant are nearby. • Reversing alarms, beacons, mirrors and cameras must be operational. • Spotter must remain visible and clear of the line of fire. • If visual or radio contact is lost, vehicle / plant movement must stop immediately. | Low | Operator / Spotter |
| Deliveries and loading / unloading | Dropped loads; vehicle interaction; obstructed road or access | High | • Plan delivery times, route, unloading area and exclusion zone before vehicle arrives. • Delivery drivers must follow site directions and remain within controlled areas. • Use spotter where reversing or manoeuvring in restricted areas. • Keep workers clear of loading / unloading zones and suspended loads. • Do not unload on public roads unless approved traffic controls are in place. | Low | Supervisor / Driver / Spotter |
| Pedestrian management | Worker or visitor struck by vehicle / plant | High | • Establish pedestrian walkways or safe routes where practicable. • Keep pedestrians separated from vehicle and plant movement areas. • Ensure visitors are escorted where required. • Use barriers, cones or signage to keep pedestrians out of traffic paths. • Workers must not use mobile phones or distractions while walking through traffic areas. | Low | Supervisor / All Workers |
| Working near public roads | Vehicle impact; public exposure; road authority breach | High | • Do not work on or adjacent to public roads without required approval, TMP / TGS and controls. • Position vehicles and plant to minimise exposure to live traffic. • Maintain safe clearances from traffic lanes. • Use accredited traffic controllers where required. • Stop work if road users do not comply or controls are ineffective. | Low | Supervisor / Traffic Controller |
| Dust, mud and poor visibility | Poor driver visibility; slip hazard; public nuisance | Medium | • Use dust suppression where vehicle or plant movements generate dust. • Clean mud or debris from roadways where it creates a slip or traffic hazard. • Use lighting, beacons or additional controls where visibility is reduced. • Stop or modify work during heavy rain, poor visibility or unsafe weather. | Low | Supervisor / All Workers |
| Night work or low light conditions | Reduced visibility; vehicle collision | High | • Adequate lighting must be provided where work occurs in low light. • Use reflective PPE and reflective signs / cones where required. • Ensure lights do not create glare for drivers or operators. • Increase separation distances or controls where visibility is reduced. • Stop work if workers or controls cannot be clearly seen. | Low | Supervisor / All Workers |
| Emergency access and incident response | Blocked emergency route; delayed rescue | High | • Emergency vehicle access must remain clear at all times. • Confirm assembly point and evacuation routes at pre-start. • Traffic controls must allow emergency vehicles to enter and exit when required. • In an incident, stop work, make area safe and contact emergency services where required. • Preserve the scene for serious incidents where safe to do so. | Low | Supervisor / All Workers |
| Monitoring traffic controls | Controls moved; changed site conditions; road user confusion | Medium | • Supervisor or traffic controller to inspect traffic controls regularly. • Replace damaged, fallen, dirty or moved signs / cones immediately. • Adjust controls if plant movements, deliveries or site conditions change. • Workers must report near misses, public complaints, damaged controls or unsafe traffic behaviour. • Do not continue work if controls are ineffective. | Low | Supervisor / Traffic Controller |
| Completion and pack up | Premature removal of controls; poor housekeeping; remaining hazards | Medium | • Remove tools, materials, rubbish and trip hazards from the work area. • Remove traffic controls only when the work area is safe and workers are clear of traffic exposure. • Pack up signs and cones in safe sequence away from live traffic where practicable. • Ensure site access, roads and emergency routes are left clear. • Close permits, inspections and records where required. | Low | Supervisor / All Workers |

ENVIRONMENTAL AND SITE CONTROLS

Dust suppression must be used where vehicle or plant movement generates dust.

Mud, debris or spoil must not be tracked onto public roads or access ways.

Spill kits must be available where plant, fuel, oil or chemicals are present.

Noise and reversing alarms must be managed in accordance with site requirements while maintaining safety.

Waste, damaged signs, cones and packaging must be disposed of correctly.

Traffic controls must not block drains, waterways or emergency access.

Environmental incidents must be reported immediately to John Austen / Site Supervisor.

PROJECT INFORMATION AND REQUIREMENTS

| Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |
| --- | --- |
| Workers | 2+ or as required by task |
| Competency | General Safety Induction Card, site induction, task instruction, operator competency / VOC, traffic controller / TMI competency where required |
| Training | Austen Site Specific Induction, toolbox talks, daily pre-starts and Energy Queensland / site inductions as required |
| Supervision & Monitoring | John Austen / project supervisor to monitor traffic controls, access routes and worker separation |
| Plant & Equipment | Excavator, bobcat, trucks, rollers, delivery vehicles, light vehicles and other project specific plant |
| Tools and Equipment | Traffic signs, cones, bollards, barricades, radios, beacons, lights, stop/slow bats, spill kits and first aid equipment |
| Chemicals | Diesel, petrol, oil, hydraulic fluid or other substances associated with plant and vehicle use |
| PPE | Refer to PPE section and site requirements |
| SWMS Review | Reviewed by John Austen during works and updated when conditions change |
| Legislation & Codes of Practice | WHS Act 2011; WHS Regulation 2011; Managing Risks of Plant in the Workplace 2021; How to Manage WHS Risks 2021; Managing the Work Environment and Facilities 2021; Manual Tasks Code of Practice 2021; relevant road authority and traffic management requirements; Environmental Protection Act 1994; Biosecurity Act 2014 |

EMERGENCY AND INCIDENT RESPONSE

Stop work immediately.

Make the area safe if it is safe to do so.

Move workers clear of traffic or plant movement areas.

Contact Emergency Services on 000 for serious injury, traffic incident, fire or rescue response.

Notify John Austen / Site Supervisor and the principal contractor immediately.

Preserve the incident scene where required and safe to do so.

Maintain emergency vehicle access and direct emergency services to the site if required.

Report all incidents, hazards, near misses, property damage and public interface issues immediately.

Do not restart work until the hazard is controlled and the SWMS / traffic controls have been reviewed if required.

WORKER SIGN ON

We, the undersigned, confirm this Safe Work Method Statement has been explained, its contents are clearly understood and we will comply with the stated requirements. We also confirm that we have the required qualifications to undertake these activities.

| Name | Signature | Date |
| --- | --- | --- |

SUPERVISOR DECLARATION

I confirm I have trained all above workers in this SWMS and I hereby confirm that the documented precautions, controls and work methods will be complied with.

| Name | Position | Date | Signature |
| --- | --- | --- | --- |
| John Austen | Site Supervisor |  |  |

THINK SAFE - WORK SAFE - GO HOME SAFE

Austen Landscaping & Earthmoving | ABN 30 643 517 350 | Phone: 0407 763 435 | Email: kathieshome@westnet.com.au | MLCH-07 | Uncontrolled when printed