|  | SAFE WORK METHOD STATEMENT (SWMS) MLCH-08 - SILICA & DUST EXPOSURE |
| --- | --- |

This SWMS must be reviewed with all workers before work commences. Work must stop immediately if controls cannot be maintained or conditions change.

| Activity / Task | Silica and Dust Exposure Control | Location | MILC - Millchester Substation / Project Specific |
| --- | --- | --- | --- |
| Scope | Applies to all Austen workers performing or working near dust-generating activities including cutting, grinding, drilling, excavation, sweeping, loading, unloading and clean-up. | SWMS No. | MLCH-08 |
| Training & Experience | Competent, trained and site inducted workers. Workers using dust-generating tools must understand RCS controls, PPE and exclusion requirements. | Prepared By | John Austen |
| Site Risk Assessment By | Site Supervisor: John Austen | Creation / Review Date | 15/05/2026 |
| Revision | A | Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |

PURPOSE

This SWMS sets out the hazards, risks and controls for respirable crystalline silica (RCS), construction dust and airborne contaminants generated during civil, earthworks, concrete, masonry, excavation, sweeping and clean-up activities. The intent is to prevent worker exposure by eliminating dust at the source where practicable and using wet methods, extraction, exclusion zones, respiratory protection and safe clean-up controls.

SCOPE

Concrete, masonry, brick, block, tile, stone or engineered product cutting, drilling, grinding or chasing.

Excavation, trenching, backfilling, loading, unloading and movement of dry soil, road base or spoil.

Use of power tools, saws, grinders, jackhammers, drills, compactors, blowers or sweepers that may generate dust.

Clean-up of dusty work areas, slurry, spoil and settled dust.

Workers assisting, spotting, supervising or working nearby dust-generating activities.

CRITICAL RISKS AND MANDATORY CONTROLS

| Critical Risk | Possible Outcome | Mandatory Controls |
| --- | --- | --- |
| Respirable crystalline silica exposure | Silicosis, lung disease, cancer or respiratory illness | Use wet cutting / dust suppression; use on-tool extraction where available; wear P2/P3 respiratory protection as required; keep workers out of dust plume. |
| Dry cutting, grinding or drilling | High dust exposure | Avoid dry cutting where practicable. Use wet methods, vacuum extraction, suitable tools and exclusion zones. |
| Dust from excavation or plant movement | Worker exposure, poor visibility, public nuisance | Use water cart / hose suppression, reduce speed, manage stockpiles and stop work if dust cannot be controlled. |
| Incorrect respiratory protection | Uncontrolled inhalation exposure | Use task-suitable respirator, fit check before use, clean-shaven seal where tight-fitting mask is used, maintain and store correctly. |
| Poor housekeeping / dry sweeping | Re-suspension of dust and exposure | Use wet clean-up or HEPA vacuum. Do not dry sweep, blow down with compressed air or use leaf blowers on silica dust. |
| Dust entering vehicles / site offices | Secondary exposure | Keep doors/windows closed, remove dusty PPE before entry where practicable, clean surfaces using wet methods or HEPA vacuum. |
| Dust affecting visibility | Vehicle collision, plant strike or public risk | Stop or modify work if visibility is reduced. Increase separation, water suppression and traffic controls. |
| Contaminated or unknown material | Exposure to asbestos, lead, chemicals or biological hazards | Stop work if suspect material is identified. Isolate area and seek direction from supervisor / principal contractor. |
| Noise from cutting / grinding | Hearing damage | Use hearing protection and manage exposure duration. |
| Slurry / runoff from wet methods | Slip hazard or environmental harm | Contain slurry, protect drains and dispose of waste correctly. |

HAZARD IDENTIFICATION

| Hazard Group | Hazards Identified |
| --- | --- |
| Airborne Contaminants | Respirable crystalline silica, nuisance dust, concrete dust, soil dust, fumes and particles. |
| Tools and Equipment | Concrete saws, grinders, drills, jackhammers, compactors, sweepers, blowers, plant and trucks. |
| Mechanical / Kinetic | Flying particles, fragments, kickback, struck-by hazards and moving plant. |
| Noise / Vibration | Hearing damage, hand-arm vibration and fatigue from powered tools or plant. |
| Electrical | Use of powered tools, leads, generators and water near electrical equipment. |
| Gravity | Slips/trips from hoses, leads, slurry, rubble, offcuts and uneven surfaces. |
| Manual Handling | Handling tools, hoses, saws, blocks, bags, spoil and awkward materials. |
| Environmental | Slurry runoff, sediment, dust migration, contaminated waste and waterway impact. |
| Heat / UV | Heat stress and dehydration from outdoor work and respiratory PPE use. |

HIGH RISK WORK INTERFACE

Silica and dust work may interface with powered mobile plant, excavation, traffic management, working near underground services, work near electrical installations, manual handling and environmental controls. Relevant SWMS must also be reviewed where these risks apply.

PPE REQUIREMENTS

| PPE Item | Requirement |
| --- | --- |
| Safety helmet | Required onsite and around plant, overhead hazards or cutting areas. |
| Safety footwear | Required at all times onsite. |
| Eye protection | Medium impact eye protection. Face shield where cutting, grinding or flying particles are present. |
| Respiratory protection | P2 respirator as minimum where dust exposure is possible; P3 or half-face respirator where required by risk assessment or site direction. |
| Hearing protection | Required when using saws, grinders, jackhammers, compactors or noisy plant. |
| Task specific gloves | Required when handling tools, materials, slurry, waste or sharp materials. |
| Hi-vis long sleeve cotton clothing | Required for visibility and UV protection. |
| UV protection and 30+ sunscreen | Required for outdoor works. |
| Wet weather / waterproof PPE | Required where wet cutting or slurry handling creates splash or wet clothing risk. |

RISK RATING GUIDE

| Risk Rating | Required Action |
| --- | --- |
| Extreme | Do not proceed. Alternative method required. |
| High | Senior management / supervisor approval required before work proceeds. |
| Medium | Management responsibility must be specified and controls monitored. |
| Low | Work may proceed with normal supervision and routine controls. |
| Stop Work | All workers have authority to stop work if dust controls cannot be maintained. |

SEQUENCE OF WORK AND CONTROL MEASURES

| Sequence of Work | Hazards / Risk | Initial Risk | Work Methods & Control Measures | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning, preparation and competency | Untrained workers; incorrect method; uncontrolled dust exposure | High | • All workers must hold current construction induction and complete site induction. • Identify materials to be cut, drilled, ground, disturbed or cleaned. • Confirm whether material may contain crystalline silica, asbestos, lead or other contaminants. • Select the method that produces the least dust. • Review relevant SWMS including MLCH-01, MLCH-04, MLCH-05, MLCH-06 and MLCH-07 where applicable. • Confirm controls, PPE, emergency response and waste disposal before work starts. | Low | Supervisor |
| Pre-start and consultation | Poor communication; changed conditions; workers unaware of dust risk | Medium | • Attend Austen daily pre-start meeting before commencing work. • Discuss dust-generating tasks, exclusion zones, wet methods, respirator requirements, wind direction and clean-up controls. • Confirm who is operating tools, managing water/extraction and monitoring dust. • Stop work if dust becomes uncontrolled or the task changes. | Low | Supervisor / All Workers |
| Material identification | Unknown silica or contaminated material | High | • Review SDS, product information, drawings or site information where available. • Treat concrete, brick, block, tile, stone, engineered stone and similar products as silica-containing unless confirmed otherwise. • Stop work if asbestos, lead paint, contaminated soil or unknown material is suspected. • Isolate the area and notify supervisor / principal contractor before disturbing suspect material. | Low | Supervisor / All Workers |
| Set up work area and exclusion zone | Workers exposed to dust plume; public exposure; poor visibility | High | • Establish exclusion zone around dust-generating work. • Keep non-essential workers out of the work area. • Consider wind direction, traffic, public interface, adjacent workers and nearby buildings or vehicles. • Use barriers, cones, signage and spotters where required. • Ensure emergency access and escape routes remain clear. | Low | Supervisor / All Workers |
| Tool and equipment inspection | Faulty tools; failed controls; electrical hazards | Medium | • Inspect tools, guards, blades, hoses, water supply, extraction units, leads and generators before use. • Confirm water suppression or on-tool extraction is working before cutting or grinding. • Electrical tools and leads must be tested/tagged and kept clear of water pooling. • Remove defective equipment from service until repaired. | Low | Operator / Supervisor |
| Respiratory protection | Incorrect mask; poor seal; inhalation exposure | High | • Select respirator suitable for the task and dust level. • P2 respirator is minimum where dust exposure is possible unless higher protection is required. • Fit check tight-fitting respirator before use. • Workers must be clean-shaven where a tight seal is required. • Replace disposable masks when damaged, wet, dirty or hard to breathe through. • Store reusable respirators clean and protected from dust. | Low | All Workers |
| Wet cutting / drilling / grinding | Silica dust exposure; slurry; electric shock | High | • Wet methods must be used where practicable. • Confirm water flow reaches the cutting / grinding point before starting. • Manage slurry to prevent slips, runoff or drain contamination. • Keep electrical leads and connections away from water pooling. • Stop work if water suppression fails or dust becomes visible and uncontrolled. | Low | Operator / Supervisor |
| On-tool extraction / HEPA vacuum | Dust release from failed extraction | High | • Use on-tool extraction or HEPA vacuum where available and suitable. • Check filters, hoses, seals and collection units before use. • Do not use domestic vacuums for silica dust. • Empty dust collection units using controls that prevent dust release. • Wear respirator during emptying or filter change as required. | Low | Operator |
| Dry cutting or grinding | High silica dust exposure | High | • Dry cutting or grinding must be avoided where practicable. • If dry cutting is unavoidable, supervisor approval and additional controls are required. • Use exclusion zones, respiratory protection, extraction and reduced exposure time. • Keep other workers upwind and away from dust plume. • Stop work if dust cannot be controlled. | Low | Supervisor / Operator |
| Excavation, plant movement and dusty ground | Dust plume, poor visibility, traffic or plant interaction | Medium | • Use water cart, hose or other dust suppression where plant movement or excavation generates dust. • Reduce vehicle speed on dry access roads. • Manage stockpiles, dry spoil and exposed ground to limit dust. • Stop or modify work if visibility is reduced or dust leaves the controlled work area. • Coordinate with MLCH-04 and MLCH-07 where plant or traffic interface exists. | Low | Supervisor / All Workers |
| Manual handling of dusty materials | Dust inhalation; strain; spills | Medium | • Use mechanical aids or team lifting for heavy bags, blocks, pavers or materials. • Avoid dropping dusty materials. • Open bags carefully and close or cover unused materials. • Store materials securely and at safe working heights where practicable. • Follow MLCH-01 manual handling controls. | Low | All Workers |
| Housekeeping during works | Settled dust re-suspended; slips and trips | Medium | • Keep work areas clear of offcuts, slurry, dust, rubble and trip hazards. • Do not dry sweep silica dust. • Do not blow dust with compressed air or leaf blowers. • Use wet clean-up methods or HEPA vacuum. • Keep hoses and leads routed to avoid trip hazards. | Low | Supervisor / All Workers |
| Waste, slurry and disposal | Environmental harm; dust re-release; contaminated waste | Medium | • Collect slurry, dust and waste using approved methods. • Prevent slurry and sediment entering drains, waterways or sensitive areas. • Bag, cover or dampen dusty waste before transport or disposal. • Dispose of waste in approved bins or disposal areas. • Contaminated waste must be managed under site environmental requirements. | Low | Supervisor / All Workers |
| Worker hygiene and exposure reduction | Secondary exposure; dust carried into vehicles or lunch areas | Medium | • Wash hands and face before eating, drinking or leaving site where dust exposure has occurred. • Do not eat, drink or smoke in dusty work areas. • Remove or clean dusty PPE before entering vehicles, crib rooms or offices where practicable. • Do not take heavily contaminated clothing into clean areas. | Low | All Workers |
| Task monitoring | Controls fail; dust becomes uncontrolled; changed conditions | Medium | • Supervisor to monitor work area, dust plume, wind direction and control effectiveness. • Workers must report failed water suppression, extraction faults or dust issues immediately. • Review controls if weather, wind direction, task or material changes. • Stop work if dust cannot be controlled or workers are exposed. | Low | Supervisor / All Workers |
| Completion and clean up | Residual dust; poor housekeeping; uncontrolled waste | Medium | • Clean work area using wet methods or HEPA vacuum. • Remove tools, waste, slurry and trip hazards. • Check drains, access ways and surrounding areas are clean and safe. • Store respirators, tools and PPE correctly. • Close permits / records and brief supervisor on any issues. | Low | Supervisor / All Workers |
| Emergency / uncontrolled exposure | Significant dust release; worker illness; eye injury | High | • Stop work immediately. • Move workers away from dust plume and make area safe. • Provide first aid for eye exposure or breathing difficulty. • Notify John Austen / Site Supervisor. • Review controls before recommencing work. • Seek medical advice where worker symptoms or significant exposure occurs. | Low | Supervisor / All Workers |

ENVIRONMENTAL AND SITE CONTROLS

Dust suppression must be used where dust is generated.

Slurry, sediment and dusty waste must be contained and disposed of in accordance with site requirements.

Drains, waterways and sensitive areas must be protected from slurry, sediment and dust.

Stockpiles and exposed soil should be dampened, covered or stabilised where required.

Contaminated material, asbestos or unknown substances must be reported and managed under specific controls.

Environmental incidents or dust leaving the work area must be reported immediately to John Austen / Site Supervisor.

PROJECT INFORMATION AND REQUIREMENTS

| Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |
| --- | --- |
| Workers | 2+ or as required by task |
| Competency | General Safety Induction Card, site induction, task instruction, power tool competency where required and respirator instruction / fit check requirements. |
| Training | Austen Site Specific Induction, toolbox talks, daily pre-starts and Energy Queensland / site inductions as required. |
| Supervision & Monitoring | John Austen / project supervisor to monitor dust controls, PPE use, exclusion zones and clean-up. |
| Plant & Equipment | Concrete saw, grinder, drill, jackhammer, compactor, excavator, water cart, hose, HEPA vacuum, extraction equipment and project specific tools. |
| Tools and Equipment | Barriers, signage, water supply, dust extraction, HEPA vacuum, bins, bags, wet clean-up equipment, first aid and eye wash. |
| Chemicals | Diesel, petrol, oil, hydraulic fluid or other substances associated with plant use. Refer SDS for cutting fluids or additives if used. |
| PPE | Refer to PPE section and site requirements. |
| SWMS Review | Reviewed by John Austen during works and updated when conditions change. |
| Legislation & Codes of Practice | WHS Act 2011; WHS Regulation 2011; Managing Respirable Crystalline Silica Dust Exposure in Construction and Manufacturing of Construction Elements Code of Practice 2022; Managing Risks of Hazardous Chemicals in the Workplace 2021; Hazardous Manual Tasks Code of Practice 2021; Managing Noise and Preventing Hearing Loss at Work 2021; How to Manage WHS Risks 2021; Environmental Protection Act 1994; Biosecurity Act 2014. |

EMERGENCY AND INCIDENT RESPONSE

Stop work immediately if dust controls fail or uncontrolled exposure occurs.

Move workers away from dust plume and make the area safe if safe to do so.

Provide first aid for eye irritation, breathing difficulty, injury or heat stress.

Use eyewash if dust or slurry enters eyes and seek medical advice if symptoms persist.

Notify John Austen / Site Supervisor and principal contractor immediately for significant exposure, injury or environmental incident.

Contact Emergency Services on 000 for serious injury, breathing difficulty or emergency response.

Do not restart work until controls have been reviewed, reinstated and workers re-briefed.

WORKER SIGN ON

We, the undersigned, confirm this Safe Work Method Statement has been explained, its contents are clearly understood and we will comply with the stated requirements. We also confirm that we have the required qualifications to undertake these activities.

| Name | Signature | Date |
| --- | --- | --- |

SUPERVISOR DECLARATION

I confirm I have trained all above workers in this SWMS and I hereby confirm that the documented precautions, controls and work methods will be complied with.

| Name | Position | Date | Signature |
| --- | --- | --- | --- |
|  | Site Supervisor |  |  |

THINK SAFE - WORK SAFE - GO HOME SAFE

Austen Landscaping & Earthmoving | ABN 30 643 517 350 | Phone: 0407 763 435 | Email: kathieshome@westnet.com.au | MLCH-08 | Uncontrolled when printed