|  | SAFE WORK METHOD STATEMENT (SWMS) MLCH-09 - USE OF POWER TOOLS |
| --- | --- |

This SWMS must be reviewed with all workers before work commences. Work must stop immediately if controls cannot be maintained or conditions change.

| Activity / Task | Use of Power Tools and Small Powered Equipment | Location | MILC - Millchester Substation / Project Specific |
| --- | --- | --- | --- |
| Scope | Applies to all Austen workers using, assisting with, inspecting, maintaining or working near portable power tools and small powered equipment. | SWMS No. | MLCH-09 |
| Training & Experience | Competent, trained and site inducted workers. Task-specific training required for high risk tools or specialist equipment. | Prepared By | John Austen |
| Site Risk Assessment By | Site Supervisor: John Austen | Creation / Review Date | 15/05/2026 |
| Revision | A | Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |

PURPOSE

This SWMS sets out the hazards, risks and controls for the safe use of portable power tools and small powered equipment. It is designed for civil, earthworks, utility and substation work environments where workers may use grinders, drills, saws, cut-off tools, jackhammers, compactors, generators, leads, battery tools and associated equipment.

SCOPE

Inspection, use and storage of portable power tools.

Use of electric, battery, pneumatic, petrol and hydraulic tools.

Work involving cutting, grinding, drilling, hammering, fastening and minor fabrication.

Use of extension leads, generators, chargers and battery packs.

Work near mobile plant, excavations, underground services, live electrical assets or public access areas.

Set up, operation, monitoring, clean up and incident response.

CRITICAL RISKS AND MANDATORY CONTROLS

| Critical Risk | Possible Outcome | Mandatory Controls |
| --- | --- | --- |
| Electric shock | Shock, burns, electrocution or fatality | Use tested and tagged electrical equipment; inspect leads/tools before use; use RCD protection; keep tools dry; remove damaged tools from service. |
| Contact with moving parts | Cuts, amputations, lacerations or entanglement | Guards fitted and used; keep hands clear; secure workpiece; do not bypass safety devices; isolate before changing blades/discs. |
| Flying particles / projectiles | Eye injury, facial injury or cuts | Wear eye protection / face shield; use correct disc/blade; inspect accessories; establish exclusion zones where required. |
| Dust and silica exposure | Respiratory illness or long-term lung disease | Use wet cutting, dust extraction or HEPA vacuum where required; wear P2/P3 respirator where required; follow MLCH-08 controls. |
| Noise and vibration | Hearing loss, hand-arm vibration or fatigue | Use hearing protection; limit exposure time; rotate tasks; maintain tools and select low-vibration equipment where practicable. |
| Fire / hot works | Fire, burns or property damage | Hot work permit where required; remove flammables; keep fire extinguisher available; allow hot surfaces to cool. |
| Incorrect tool selection | Tool failure, injury or poor control | Use the right tool for the task; follow manufacturer instructions; operators must be competent and fit for work. |
| Stored energy / batteries | Fire, explosion, chemical exposure or tool start-up | Use approved chargers; inspect batteries; isolate tools before adjustment; do not use damaged batteries. |
| Working near live services | Electrocution, arc flash or service damage | Review MLCH-02, MLCH-03 and MLCH-05. Maintain exclusion zones and do not use conductive tools near live parts unless authorised and controlled. |
| Poor housekeeping | Slips, trips, falls or damaged leads | Keep leads clear; use lead stands or covers where required; clean as you go; store tools safely. |

HAZARD IDENTIFICATION

| Hazard Group | Hazards Identified |
| --- | --- |
| Electrical | Electric shock, damaged leads, faulty tools, wet conditions, generator use, battery charging. |
| Mechanical | Moving blades, discs, bits, rotating parts, pinch points, kickback, entanglement. |
| Kinetic | Flying particles, projectiles, broken discs/blades, uncontrolled tool movement. |
| Noise / Vibration | Hearing damage, hand-arm vibration, fatigue and reduced communication. |
| Hazardous Substances | Dust, silica, fumes, fuel, oils, battery chemicals and cleaning agents. |
| Thermal | Hot surfaces, sparks, burns, fire or hot work exposure. |
| Muscular | Manual handling of tools, awkward posture, repetitive force and overreaching. |
| Work Area | Poor access, trip hazards, wet surfaces, confined work area, poor lighting and public interface. |
| Environmental | Dust, slurry, waste discs/blades, sparks, spills and damaged materials. |

HIGH RISK WORK INTERFACE

Use of power tools may interface with high risk construction work including work near electrical installations/services, excavation, silica exposure, hot work, powered mobile plant, traffic interfaces, confined or restricted areas and live substations. Relevant task-specific SWMS must also be reviewed where these risks apply.

PPE REQUIREMENTS

| PPE Item | Requirement |
| --- | --- |
| Safety helmet | Required onsite and where overhead, plant or falling-object hazards exist. |
| Safety footwear | Required at all times onsite. |
| Medium impact eye protection | Required when using or working near power tools. Face shield required for grinding, cutting or projectile risk. |
| Task specific gloves | Required where safe for task; do not wear loose gloves near rotating equipment where entanglement risk exists. |
| Hearing protection | Required when using noisy tools or working near noisy equipment. |
| Respiratory protection | P2/P3 respirator required where dust, silica or fumes are not adequately controlled. |
| Hi-vis long sleeve cotton clothing | Required for visibility and reduced synthetic clothing risk. |
| Arc flash / electrical PPE | Required where electrical risk or client requirement applies. |
| UV protection and sunscreen | Required for outdoor works. |

RISK RATING GUIDE

| Risk Rating | Required Action |
| --- | --- |
| Extreme | Do not proceed. Alternative method required. |
| High | Senior management / supervisor approval required before work proceeds. |
| Medium | Management responsibility must be specified and controls monitored. |
| Low | Work may proceed with normal supervision and routine controls. |
| Stop Work | All workers have authority to stop work if controls cannot be maintained. |

SEQUENCE OF WORK AND CONTROL MEASURES

| Sequence of Work | Hazards / Risk | Initial Risk | Work Methods & Control Measures | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning, preparation and competency | Untrained workers; incorrect tool selection; no permit | High | • All workers must hold current construction induction and complete site induction. • Workers must be trained and competent in the tool being used. • Supervisor to confirm permits, task requirements, PPE, exclusion zones and work method. • Review MLCH-08 for dust/silica, MLCH-03 for live electrical work and MLCH-05/02 where excavation or underground services are involved. • Confirm emergency response, first aid and fire controls before work starts. | Low | Supervisor |
| Pre-start and consultation | Poor communication; changed conditions; unclear task | Medium | • Attend Austen daily pre-start meeting before commencing work. • Discuss tools to be used, hazards, guarding, dust controls, noise, exclusion zones and emergency response. • Identify additional hazards and implement controls before work starts. • Stop work if the task changes or controls cannot be maintained. | Low | Supervisor / All Workers |
| Tool selection | Incorrect tool; uncontrolled energy; equipment failure | High | • Select the correct tool for the job and material. • Follow manufacturer instructions and site requirements. • Do not use makeshift tools or modify safety controls. • Ensure accessories are compatible with tool speed, size and rating. • Use battery, electric, pneumatic, petrol or hydraulic tools only as intended. | Low | Supervisor / All Workers |
| Inspect tool before use | Damaged tool, lead, guard, disc or battery | High | • Inspect tools before each use. • Check guards, handles, switches, leads, plugs, batteries, discs, blades and attachments. • Electrical tools, leads and RCDs must be tested/tagged where required by site controls. • Remove defective or damaged equipment from service and tag out. • Do not use tools with missing guards, damaged leads or faulty switches. | Low | All Workers |
| Electrical safety and RCD protection | Electric shock; wet conditions; damaged leads | High | • Use RCD protection for portable electrical equipment where required. • Keep electrical tools and leads dry and protected from damage. • Keep leads clear of water, sharp edges, vehicle paths and trip areas. • Use lead stands, covers or elevated leads where practicable. • Do not use electrical tools in wet conditions unless suitable controls are in place. | Low | Supervisor / All Workers |
| Battery tool and charger safety | Battery fire; overheating; chemical exposure | Medium | • Use manufacturer approved batteries and chargers. • Inspect batteries for cracks, swelling, overheating or leakage. • Do not charge batteries in direct sun, wet areas or near flammable materials. • Store batteries away from metal objects and heat sources. • Remove damaged batteries from service and report immediately. | Low | All Workers |
| Set up work area | Public exposure; trips; poor access; poor lighting | Medium | • Establish a stable work area with clear access and egress. • Barricade or signpost where projectiles, sparks, noise or dust may affect others. • Secure the workpiece before cutting, grinding or drilling. • Keep cords, hoses and materials out of walkways. • Ensure adequate lighting and ventilation. | Low | Supervisor / All Workers |
| Cutting and grinding | Laceration, amputation, eye injury, disc failure, fire | High | • Guards must be fitted and correctly positioned. • Use correct disc/blade type for tool and material. • Inspect discs/blades before use and do not use damaged accessories. • Keep hands and body clear of line of fire. • Allow tool to reach full speed before contacting workpiece. • Do not force tool or twist cutting disc in the cut. • Face shield, eye protection, hearing protection and gloves as required. | Low | All Workers |
| Drilling, fastening and impact tools | Kickback, flying particles, noise, hand injury | Medium | • Secure workpiece and select correct bit/attachment. • Keep hands clear of drill path and pinch points. • Use side handle where fitted or required. • Maintain stable footing and avoid overreaching. • Wear eye and hearing protection where required. | Low | All Workers |
| Jackhammering, hammer drilling and breaking | Vibration, dust, silica, muscle strain, service strike | High | • Confirm underground services before breaking ground or slabs. • Use dust controls and respiratory protection as required. • Rotate workers and take breaks to reduce vibration exposure. • Maintain firm footing and use correct manual handling posture. • Stop work if unexpected conduits, cables, pipes or marker tape are found. | Low | Supervisor / All Workers |
| Dust and silica controls | Respirable crystalline silica exposure; poor visibility | High | • Use wet methods, on-tool extraction or HEPA vacuum where practicable. • Dry cutting/grinding of silica-containing materials is not permitted unless approved controls are in place. • Wear P2/P3 respirator where required by risk assessment. • Establish exclusion zones to prevent exposing other workers. • Follow MLCH-08 Silica & Dust Exposure controls. | Low | Supervisor / All Workers |
| Hot work / sparks | Fire, burns, damage to property or services | High | • Check whether hot work permit is required. • Remove flammable materials or protect them from sparks. • Fire extinguisher must be available. • Maintain fire watch where required. • Allow hot materials to cool before handling or storage. | Low | Supervisor / All Workers |
| Working near live electrical assets | Electrocution, arc flash, service damage | High | • Do not use conductive tools near exposed live parts unless specifically authorised and controlled. • Maintain exclusion zones and minimum approach distances. • Review MLCH-03 before working near exposed live parts. • Use ESO where required by site controls. • Stop work if electrical status is uncertain. | Low | Supervisor / ESO / All Workers |
| Use of generators and petrol-powered tools | Fire, fumes, burns, noise | High | • Use petrol-powered tools/generators in well ventilated areas only. • Do not operate petrol equipment in enclosed spaces. • Keep fuel away from ignition sources and hot surfaces. • Fire extinguisher and spill kit must be available. • Refuel only when equipment is off and cooled. | Low | Operator / Supervisor |
| Manual handling and ergonomics | Strain, fatigue, awkward posture | Medium | • Use correct lifting and carrying techniques. • Use mechanical aids or team handling for heavy tools/equipment. • Avoid prolonged awkward postures and overreaching. • Rotate tasks and take breaks for repetitive or vibrating tools. • Follow MLCH-01 controls. | Low | All Workers |
| Housekeeping during works | Trips, damaged leads, poor access, contact with sharp waste | Medium | • Keep work area clear of offcuts, leads, hoses and waste. • Place waste discs/blades, packaging and materials in correct bins. • Do not leave sharp objects exposed. • Keep emergency access clear. • Clean as you go. | Low | All Workers |
| Tool adjustment, blade/disc changes and maintenance | Unexpected start-up; cuts; stored energy | High | • Isolate tool from power source before adjusting, cleaning or changing accessories. • Remove battery or unplug tool before blade/disc changes. • Allow moving parts to stop completely before placing tool down. • Only competent persons to maintain equipment. • Do not bypass interlocks or guards. | Low | All Workers |
| Task monitoring and changed conditions | Unidentified hazards; work outside SWMS scope | Medium | • Supervisor to monitor work activities and controls. • All workers to monitor for new hazards. • Review SWMS if conditions change, tool type changes or additional risks are identified. • Stop work if controls cannot be maintained. • Do not recommence until hazards are controlled and workers have been briefed. | Low | Supervisor / All Workers |
| Completion and pack up | Poor housekeeping; damaged tools; unsafe storage | Medium | • Switch off and isolate tools. • Clean tools and inspect for damage before storage. • Store tools, leads, batteries and accessories safely. • Remove waste, offcuts and trip hazards. • Report defects and return permits / records where required. | Low | Supervisor / All Workers |

ENVIRONMENTAL AND SITE CONTROLS

Dust suppression and extraction must be used where dust is generated.

Silica-containing dust must be controlled in accordance with MLCH-08.

Waste blades, discs, packaging, offcuts and damaged tools must be disposed of correctly.

Spill kits must be available where fuel, oil or chemicals are present.

Noise must be managed while maintaining safety communication.

Sparks and hot work must not cause fire or environmental damage.

Environmental incidents must be reported immediately to John Austen / Site Supervisor.

PROJECT INFORMATION AND REQUIREMENTS

| Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |
| --- | --- |
| Workers | 2+ or as required by task |
| Competency | General Safety Induction Card, site induction, task instruction, power tool competency, hot work / silica / electrical awareness where required |
| Training | Austen Site Specific Induction, toolbox talks, daily pre-starts and Energy Queensland / site inductions as required |
| Supervision & Monitoring | John Austen / project supervisor to monitor work, inspect tools and review controls |
| Plant & Equipment | Grinders, drills, saws, jackhammers, compactors, generators, battery tools, leads, chargers and project specific tools |
| Tools and Equipment | RCDs, guards, handles, dust extraction, HEPA vacuum, water suppression, fire extinguisher, spill kit and first aid equipment |
| Chemicals | Diesel, petrol, oil, grease, battery chemicals, cutting fluids or other substances associated with tool use |
| PPE | Refer to PPE section and site requirements |
| SWMS Review | Reviewed by John Austen during works and updated when conditions change |
| Legislation & Codes of Practice | WHS Act 2011; WHS Regulation 2011; Electrical Safety Act 2002; Electrical Safety Regulation 2013; Managing Electrical Risks in the Workplace 2021; Managing Risks of Plant in the Workplace 2021; Hazardous Manual Tasks Code of Practice 2021; Managing Noise and Preventing Hearing Loss at Work 2021; Managing Respirable Crystalline Silica Dust Exposure in Construction and Manufacturing of Construction Elements 2022; How to Manage WHS Risks 2021; Environmental Protection Act 1994; Biosecurity Act 2014 |

EMERGENCY AND INCIDENT RESPONSE

Stop work immediately.

Make the area safe if it is safe to do so.

Isolate power / remove battery / shut down equipment where safe.

Notify John Austen / Site Supervisor and the principal contractor immediately.

Call Emergency Services on 000 for serious injury, electric shock, fire or rescue response.

Use first aid equipment only when safe to do so.

For electric shock, do not touch the person until the source is isolated.

Use fire extinguisher only if trained and safe to do so.

Preserve the incident scene where required.

Do not restart work until the hazard is controlled and the SWMS has been reviewed if required.

WORKER SIGN ON

We, the undersigned, confirm this Safe Work Method Statement has been explained, its contents are clearly understood and we will comply with the stated requirements. We also confirm that we have the required qualifications to undertake these activities.

| Name | Signature | Date |
| --- | --- | --- |

SUPERVISOR DECLARATION

I confirm I have trained all above workers in this SWMS and I hereby confirm that the documented precautions, controls and work methods will be complied with.

| Name | Position | Date | Signature |
| --- | --- | --- | --- |
| John Austen | Site Supervisor |  |  |

THINK SAFE - WORK SAFE - GO HOME SAFE

Austen Landscaping & Earthmoving | ABN 30 643 517 350 | Phone: 0407 763 435 | Email: kathieshome@westnet.com.au | MLCH-09 | Uncontrolled when printed