|  | SAFE WORK METHOD STATEMENT (SWMS) MLCH-10 - DELIVERY, LOADING & UNLOADING OPERATIONS |
| --- | --- |

This SWMS must be reviewed with all workers before work commences. Work must stop immediately if controls cannot be maintained or conditions change.

| Activity / Task | Delivery, Loading and Unloading Operations | Location | MILC - Millchester Substation / Project Specific |
| --- | --- | --- | --- |
| Scope | Applies to all Austen workers, delivery drivers, plant operators, spotters and subcontractors involved in deliveries, loading, unloading, material movement and load restraint activities. | SWMS No. | MLCH-10 |
| Training & Experience | Competent, trained and site inducted workers. Operators must hold relevant licence / VOC. Dogging, rigging, crane or forklift competency required where applicable. | Prepared By | John Austen |
| Site Risk Assessment By | Site Supervisor: John Austen | Creation / Review Date | 15/05/2026 |
| Revision | A | Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |

PURPOSE

This SWMS sets out the hazards, risks and controls for receiving deliveries, loading, unloading, moving materials, securing loads and managing delivery vehicles on civil, earthworks, utility and substation work sites. The intent is to prevent crush injuries, dropped loads, vehicle interactions, manual handling injuries, public exposure and damage to plant, services or materials.

SCOPE

Delivery vehicles entering, exiting and moving through site.

Loading and unloading plant, materials, equipment, pipes, conduits, cable drums, pallets, spoil, road base and general supplies.

Use of trucks, trailers, cranes, forklifts, telehandlers, excavators, loaders, bobcats and other plant for loading or unloading.

Manual handling, team lifting, load restraint, load release, storage and stacking.

Work around traffic, pedestrians, plant, live substations, underground services and restricted access areas.

Set up, operation, monitoring, clean up, documentation and emergency response.

CRITICAL RISKS AND MANDATORY CONTROLS

| Critical Risk | Possible Outcome | Mandatory Controls |
| --- | --- | --- |
| Vehicle / pedestrian interaction | Crush injury, struck-by injury or fatality | Separate people and vehicles; use exclusion zones, spotters, hi-vis PPE and positive communication. |
| Reversing delivery vehicles | Collision, crush injury or property damage | Avoid reversing where practicable. Use spotters, reversing alarms, beacons and agreed communication. |
| Dropped or unstable loads | Crush injury, impact injury or damaged equipment | Inspect load stability before releasing restraints. Establish exclusion zone and keep workers clear of line of fire. |
| Suspended loads | Fatality, crush injury or struck-by injury | No person under suspended loads. Use competent operators, approved lifting gear and exclusion zones. |
| Incorrect lifting plant or equipment | Plant failure, dropped load or rollover | Confirm plant capacity, attachment suitability, lifting points and ground conditions before lifting. |
| Load restraint release | Load shift, falling materials or crush injury | Release restraints from safe position. Check load before loosening straps, chains, gates or pins. |
| Manual handling | Back, shoulder, hand or knee injuries | Use mechanical aids, team lifts and safe storage heights. Avoid twisting, overreaching and carrying awkward loads. |
| Working near live services or substations | Electric shock, arc flash or service damage | Review MLCH-02, MLCH-03 and MLCH-05 where deliveries or plant operate near live assets or underground services. |
| Traffic / public interface | Vehicle impact, public injury or road incident | Use MLCH-07 controls, traffic management, site access control, signs and barriers where required. |
| Poor ground or access | Vehicle rollover, bogging, collision or load instability | Inspect route, unloading area and ground conditions. Use suitable access and stop work if unstable. |
| Environmental spill | Soil or water contamination | Use spill kits, inspect plant for leaks and report spills immediately. |

HAZARD IDENTIFICATION

| Hazard Group | Hazards Identified |
| --- | --- |
| Traffic / Vehicles | Delivery trucks, reversing vehicles, site access, public roads, restricted turning areas and blind spots. |
| Mobile Plant | Forklifts, cranes, excavators, loaders, bobcats, telehandlers, rollers and trucks operating near workers. |
| Gravity | Falling objects, dropped loads, unstable stacks, slipping from trays, falls from trucks or trailers. |
| Mechanical / Kinetic | Crush points, pinch points, load swing, load shift, chains, straps, gates and moving attachments. |
| Manual Handling | Lifting, carrying, pushing, pulling, awkward loads, team lifting and repetitive handling. |
| Electrical | Deliveries near substations, overhead lines, live electrical assets or underground services. |
| Environmental | Fuel/oil spills, dust, mud, runoff, waste packaging and weather impacts. |
| Communication | Unclear signals, radio failure, driver not inducted, language barriers or competing instructions. |
| Work Area | Uneven ground, slopes, soft ground, poor lighting, poor housekeeping and restricted access. |

HIGH RISK WORK INTERFACE

Delivery, loading and unloading activities may interface with high risk construction work including powered mobile plant, traffic management, work near electrical installations/services, excavation, live substations, public roads, lifting operations and underground services. Relevant SWMS must also be reviewed where these risks apply.

PPE REQUIREMENTS

| PPE Item | Requirement |
| --- | --- |
| Hi-vis long sleeve clothing | Required at all times around vehicles, plant and deliveries. |
| Safety helmet | Required onsite and when working near plant, suspended loads or loading areas. |
| Safety footwear | Required at all times onsite. |
| Medium impact eye protection | Required where dust, straps, chains, tools or loading activities may create eye risk. |
| Task specific gloves | Required when handling straps, chains, gates, pallets, materials, tools or sharp edges. |
| Hearing protection | Required where plant, trucks or equipment create noise exposure. |
| Respiratory protection | Required where dust, fumes or exhaust exposure cannot be controlled. |
| UV protection and 30+ sunscreen | Required for outdoor works. |
| Wet weather / reflective PPE | Required where visibility, weather or night / low light conditions require additional controls. |

RISK RATING GUIDE

| Risk Rating | Required Action |
| --- | --- |
| Extreme | Do not proceed. Alternative method required. |
| High | Senior management / supervisor approval required before work proceeds. |
| Medium | Management responsibility must be specified and controls monitored. |
| Low | Work may proceed with normal supervision and routine controls. |
| Stop Work | All workers have authority to stop work if controls cannot be maintained. |

SEQUENCE OF WORK AND CONTROL MEASURES

| Sequence of Work | Hazards / Risk | Initial Risk | Work Methods & Control Measures | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning, preparation and competency | Untrained workers; incorrect equipment; no delivery plan | High | • All workers must hold current construction induction and complete the site induction. • Confirm delivery details, load type, vehicle type, unloading location, plant required and exclusion zone requirements before the delivery arrives. • Operators must be competent, licensed or verified for the plant being used. • Dogging, rigging, crane, forklift or other high risk work competency required where applicable. • Review MLCH-04, MLCH-07, MLCH-01 and other relevant SWMS before work starts. | Low | Supervisor |
| Pre-start and consultation | Poor communication; unclear roles; changed conditions | Medium | • Attend Austen daily pre-start meeting before commencing work. • Discuss delivery route, access, unloading zone, exclusion zones, plant movements, spotters, load restraint and emergency access. • Confirm who is supervising, spotting, operating plant, directing drivers and communicating with delivery personnel. • Stop work if the delivery method or site conditions change. | Low | Supervisor / All Workers |
| Delivery vehicle site access | Vehicle collision; public exposure; unauthorised access | High | • Delivery drivers must report to supervisor / sign in as required. • Confirm driver understands site rules, speed limits, access route and parking / unloading location. • Use designated entry and exit points where practicable. • Keep access roads clear of obstructions, parked vehicles and pedestrians. • Use MLCH-07 traffic controls where public road or site traffic interface exists. | Low | Supervisor / Driver |
| Inspect unloading area | Unstable ground; rollover; blocked emergency access | High | • Inspect ground conditions, slope, overhead hazards, underground service risk, turning space and pedestrian routes. • Select stable, level unloading area where practicable. • Maintain emergency access, site access and evacuation routes. • Do not unload near open excavations, live services, overhead lines or restricted areas unless controls are in place. • Stop work if ground is soft, unstable or unsuitable. | Low | Supervisor / Operator |
| Establish exclusion zone | Workers struck by vehicle, load or plant | High | • Establish exclusion zone around vehicle, plant, load and unloading area. • Keep non-essential workers, visitors and public out of unloading area. • Use cones, barricades, signs, spotters or verbal controls as required. • No person to stand between vehicle and fixed object, under suspended load or in line of fire. • Maintain clear communication before any load movement. | Low | Supervisor / All Workers |
| Vehicle positioning and reversing | Crush injury, collision, blind spots | High | • Avoid reversing where practicable by planning one-way travel and turning areas. • Use spotter where reversing or manoeuvring in restricted areas. • Reversing alarms, beacons, mirrors and cameras must be operational. • Spotter must remain visible and clear of line of fire. • If visual or radio contact is lost, vehicle / plant movement must stop immediately. | Low | Driver / Spotter |
| Inspect load before releasing restraint | Load shift; falling objects; crush injury | High | • Inspect the load from a safe position before loosening straps, chains, gates, pins or restraints. • Check for signs of movement, lean, damage, broken packaging or unstable stacking. • Do not stand beside or below a load that may shift when restraints are released. • Release restraints progressively and from the safe side where possible. • Stop and reassess if load appears unstable. | Low | Supervisor / Driver / All Workers |
| Loading / unloading using plant | Dropped load; crush injury; plant collision | High | • Plant must be suitable for load weight, shape, reach, ground condition and operating environment. • Confirm lifting points, attachment rating, load capacity and stability before lifting. • Keep workers clear of loading / unloading zone. • Use spotter where visibility is restricted or workers, structures or services are nearby. • Loads must be kept low and controlled during movement. • Do not exceed plant rated capacity or manufacturer requirements. | Low | Operator / Supervisor |
| Suspended loads and lifting operations | Fatality, crush injury, dropped load | High | • No person may stand or work under suspended loads. • Lifting gear must be inspected, rated and suitable for the load. • Dogging / rigging to be completed by competent persons where required. • Tag lines to be used where required and safe to do so. • Wind, load swing, ground conditions and exclusion zones must be assessed before lifting. | Low | Supervisor / Operator / Dogger |
| Manual unloading or material handling | Back strain; hand injury; dropped material | Medium | • Assess weight, size, shape, grip, stability and travel path before manual handling. • Use mechanical aids or team lifting for heavy, long or awkward items. • Keep load close to body, avoid twisting and use smooth movements. • Wear gloves when handling sharp, rough or strapped materials. • Store materials at safe height and avoid unnecessary double handling. | Low | All Workers |
| Use of pallets, forklifts or telehandlers | Forklift collision; falling pallet; unstable load | High | • Forklift / telehandler operator must be competent and authorised. • Inspect pallet condition, load stability and fork position before lifting. • Do not lift damaged pallets unless safe alternative controls are in place. • Keep load low during travel and avoid sudden turning or braking. • Pedestrians must remain outside forklift / telehandler exclusion zone. | Low | Operator / Supervisor |
| Loading / unloading near live services or substations | Electric shock, arc flash, service damage | High | • Identify overhead and underground electrical assets before positioning vehicle or plant. • Maintain exclusion zones and minimum approach distances. • Review MLCH-02, MLCH-03 and MLCH-05 where plant, loads or vehicles operate near live assets or underground services. • Use ESO / spotter where required. • Stop work if clearance distances cannot be maintained. | Low | Supervisor / ESO / Operator |
| Load restraint after loading | Load shift, road incident, falling materials | High | • Loads must be secured before vehicle movement. • Use suitable straps, chains, binders, gates, nets or restraints for the load type. • Check restraint condition and rating. • Confirm load is stable and within vehicle limits. • Driver to complete final check before leaving site. • Do not allow unsecured loads to leave site. | Low | Driver / Supervisor |
| Dust, mud, debris and public road interface | Road hazard, public complaint, traffic incident | Medium | • Prevent mud, spoil or debris being tracked onto public roads. • Clean access tracks or road surfaces if contamination creates hazard. • Use dust suppression where loading/unloading generates dust. • Cover loads where required to prevent material loss during transport. • Coordinate with MLCH-07 traffic controls where required. | Low | Supervisor / Driver |
| Refuelling or plant fluids during loading works | Fire, spill, skin/eye contact, environmental harm | Medium | • Refuelling must be completed by competent person away from ignition sources. • Fire extinguisher and spill kit must be available. • Inspect plant and vehicles for leaks before and during work. • Contain and report spills immediately. • Contaminated waste must be disposed of correctly. | Low | Operator / Supervisor |
| Task monitoring and changed conditions | Uncontrolled hazards; work outside SWMS scope | Medium | • Supervisor to monitor delivery and unloading activities. • Workers must report changes, unstable loads, plant defects, near misses or access issues. • Stop work if exclusion zones are breached or controls are ineffective. • Review controls if delivery type, work area, weather, traffic or plant changes. • Do not recommence until hazards are controlled and workers are briefed. | Low | Supervisor / All Workers |
| Completion and clean up | Poor housekeeping; unsecured materials; blocked access | Medium | • Store materials safely and securely after unloading. • Remove packaging, straps, pallets, dunnage and trip hazards from work area. • Keep access roads, emergency access and walkways clear. • Maintain barricades and signage until area is safe. • Complete delivery records, inspections, pre-starts or permits where required. | Low | Supervisor / All Workers |

ENVIRONMENTAL AND SITE CONTROLS

Spill kits must be available where plant, fuel, oil or chemicals are present.

Loads must be secured to prevent material loss, dust, spoil or debris leaving site.

Mud, debris or spoil must not be tracked onto public roads or access ways.

Waste packaging, straps, pallets and dunnage must be disposed of correctly or stored safely.

Dust suppression must be used where loading/unloading generates dust.

Drains, waterways and sensitive areas must be protected from spills, sediment, slurry and waste.

Environmental incidents must be reported immediately to John Austen / Site Supervisor.

PROJECT INFORMATION AND REQUIREMENTS

| Principal Contractor | Energy Queensland / Ergon Energy (Project Specific) |
| --- | --- |
| Workers | 2+ or as required by task |
| Competency | General Safety Induction Card, site induction, task instruction, operator competency / VOC, forklift / crane / dogger / rigger competency where required |
| Training | Austen Site Specific Induction, toolbox talks, daily pre-starts and Energy Queensland / site inductions as required |
| Supervision & Monitoring | John Austen / project supervisor to monitor delivery area, exclusion zones, access routes and load movement |
| Plant & Equipment | Trucks, trailers, excavator, bobcat, forklift, telehandler, crane, loader and other project specific mobile plant |
| Tools and Equipment | Chains, straps, binders, tag lines, dunnage, cones, barricades, radios, signage, spill kits, fire extinguisher and first aid equipment |
| Chemicals | Diesel, petrol, oil, grease, hydraulic fluid or other substances associated with plant and vehicle use |
| PPE | Refer to PPE section and site requirements |
| SWMS Review | Reviewed by John Austen during works and updated when conditions change |
| Legislation & Codes of Practice | WHS Act 2011; WHS Regulation 2011; Managing Risks of Plant in the Workplace 2021; Hazardous Manual Tasks Code of Practice 2021; Managing Noise and Preventing Hearing Loss at Work 2021; How to Manage WHS Risks 2021; Traffic management and load restraint requirements applicable to the task; Environmental Protection Act 1994; Biosecurity Act 2014 |

EMERGENCY AND INCIDENT RESPONSE

Stop work immediately.

Make the area safe if it is safe to do so.

Move workers away from unstable loads, moving vehicles, plant and line-of-fire areas.

Do not attempt to catch, hold or stop a falling or shifting load.

Contact Emergency Services on 000 for serious injury, traffic incident, fire or rescue response.

Notify John Austen / Site Supervisor and the principal contractor immediately.

Use spill kit and protect drains / waterways if safe to do so.

Preserve the incident scene where required and safe to do so.

Report all incidents, hazards, near misses, property damage, load failures and public interface issues immediately.

Do not restart work until the hazard is controlled and the SWMS has been reviewed if required.

WORKER SIGN ON

We, the undersigned, confirm this Safe Work Method Statement has been explained, its contents are clearly understood and we will comply with the stated requirements. We also confirm that we have the required qualifications to undertake these activities.

| Name | Signature | Date |
| --- | --- | --- |

SUPERVISOR DECLARATION

I confirm I have trained all above workers in this SWMS and I hereby confirm that the documented precautions, controls and work methods will be complied with.

| Name | Position | Date | Signature |
| --- | --- | --- | --- |
| John Austen | Site Supervisor |  |  |

THINK SAFE - WORK SAFE - GO HOME SAFE

Austen Landscaping & Earthmoving | ABN 30 643 517 350 | Phone: 0407 763 435 | Email: kathieshome@westnet.com.au | MLCH-10 | Uncontrolled when printed